O&M Base Requirements – Port of Eastholm

Harrow Bay Offshore Wind

David Chen  ·  12 August 2026  ·  HB-OPS-SPC-0003

# 1. Location

The O&M base will be at the Port of Eastholm, which is also the construction base for Harrow Bay. The site is 41 nautical miles from the centre of the wind farm, about 1 h 45 min transit for a 25-knot crew transfer vessel.

# 2. Building and yard

| Requirement | Specification |
| --- | --- |
| Office space | 650 m², 40 workstations, marine coordination room with view of the pontoon |
| Workshop | 400 m², 5 t overhead crane, HV test area |
| Warehouse | 1,200 m² indoor + 2,500 m² laydown yard |
| Welfare | Changing rooms for 60 technicians, drying room for survival suits, canteen |
| Training | Room for 16 people, GWO refresher area |
| Parking | 70 cars, 10 EV charging points |

# 3. Marine

| Requirement | Specification |
| --- | --- |
| Pontoon | 3 CTV berths, 24/7 access, minimum depth 3.5 m at lowest astronomical tide |
| Bunkering | Fuel and fresh water at the pontoon |
| Quay | Access for an SOV (90 m) for crew change and loading, 2 days per fortnight |
| Lifting | Mobile crane access to the pontoon for parts up to 8 t |

# 4. Lease

Heads of terms were agreed with the port authority on 21 August 2026: 15-year lease from 1 April 2027 with two 5-year extension options. The base building will be built by the port and fitted out by Weyland.