Monopile Drivability Assessment

Harrow Bay Offshore Wind

Sofie Lindqvist  ·  16 May 2025  ·  HB-GEO-TN-0014

# 1. Purpose

Confirm that the 9.5 m monopiles can be driven to design embedment with the IHC S-4000 hammer from MV Sea Atlas, and define hammer energy limits for noise and pile run control. Prepared by Oliver Grant.

# 2. Method

1D wave equation analysis (GRLWEAP) with soil resistance to driving (SRD) per Alm & Hamre, calibrated with the CPT profiles of all 40 positions. Upper-bound and best-estimate SRD were analysed.

# 3. Results

| Province | Max blow count (bl/0.25 m) | Max hammer energy (kJ) | Est. driving time (h) | Fatigue damage from driving |
| --- | --- | --- | --- | --- |
| A | 260 | 3,600 | 6.5 | 0.09 |
| B | 190 | 3,100 | 5.2 | 0.06 |
| C | 140 | 2,600 | 4.8 | 0.05 |
| D | 120 | 2,400 | 4.4 | 0.04 |

All piles are drivable to design embedment with the best-estimate SRD. For three positions in province A (WTG 03, 07, 11) the upper-bound SRD exceeds the refusal criterion of 250 bl/0.25 m in the last 2 m. Mitigation: relief drilling equipment on standby on the MV Sea Atlas for the first piles in province A.

# 4. Noise

Soft start at 10 % energy for 30 minutes; maximum hammer energy 3,600 kJ with double big bubble curtain. Predicted single-strike sound exposure level at 750 m: 158 dB re 1 µPa²s – below the consent threshold.