Incident Investigation Report HSE-INC-2026-018

Dropped object near miss on MV Sea Atlas, 3 June 2026

Hannah Weber  ·  12 June 2026  ·  HSE-INC-2026-018

| Field | Details |
| --- | --- |
| Incident no. | HSE-INC-2026-018 |
| Date / time | 3 June 2026, 10:47 |
| Location | Main deck, MV Sea Atlas, at HB-A08 |
| Classification | Near miss – high potential (dropped object) |
| Injuries | None |
| Investigation lead | Yusuf Demir (Weyland) with Atlas Marine HSE Manager |
| Report approved | Hannah Weber, 19 June 2026 |

# 1. Summary

During rigging for the lift of the transition piece for HB-A08, a shackle pin (2.8 kg) fell approximately 18 m from the crane block onto the main deck. It landed 3 m from a deck crew member who was outside the marked drop zone. Nobody was injured. Using the DROPS calculator, a 2.8 kg object falling 18 m is in the fatal category. The event was therefore classified as a high-potential near miss.

Rigging area on the main deck of MV Sea Atlas (reconstruction after the event)

# 2. Sequence of events

# 3. Root cause analysis

Preliminary finding: missing split pin on a bow shackle. ICAM workshop scheduled for 15 June.

# 4. Immediate actions

All shackles quarantined and inspected

Lifting resumed after 34 h stand-down